IP Addressing and Subnetting Assignment

- Key Differences Between IPv4 and IPv6 Addressing

| IPv4 | IPv6 |
| --- | --- |
| Uses 32-bit addresses. | Uses 128-bit addresses. |
| Written in decimal, e.g. 192.168.1.1. | Written in hexadecimal, e.g. 2001:db8::1. |
| Provides about 4.3 billion addresses. | Provides an extremely large address space. |
| Supports broadcast communication. | Does not use broadcast; uses multicast/anycast. |
| NAT is commonly used. | NAT is generally not required for address conservation. |
| Manual configuration or DHCP can be used. | Supports SLAAC, DHCPv6, and manual configuration. |

- Static IPv4 Address Assignment for Home WiFi Devices

Network: 192.168.10.0/24

Subnet Mask: 255.255.255.0

Default Gateway: 192.168.10.1

| Device | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- |
| Laptop | 192.168.10.10 | 255.255.255.0 | 192.168.10.1 |
| Phone | 192.168.10.20 | 255.255.255.0 | 192.168.10.1 |
| Smart TV | 192.168.10.30 | 255.255.255.0 | 192.168.10.1 |
| Printer | 192.168.10.40 | 255.255.255.0 | 192.168.10.1 |

- /27 Subnetting on the 192.168.1.0 Network

CIDR: /27 = Subnet Mask 255.255.255.224

Borrowed subnet bits from a /24 network: 27 - 24 = 3

Number of subnets: 2³ = 8

Addresses per subnet: 2⁵ = 32

Usable hosts per subnet: 32 - 2 = 30

Block size: 256 - 224 = 32

| Subnet | Network Address | Usable Host IPs | Broadcast Address |
| --- | --- | --- | --- |
| 1 | 192.168.1.0 | 192.168.1.1 - 192.168.1.30 | 192.168.1.31 |
| 2 | 192.168.1.32 | 192.168.1.33 - 192.168.1.62 | 192.168.1.63 |
| 3 | 192.168.1.64 | 192.168.1.65 - 192.168.1.94 | 192.168.1.95 |

- Network Details for 10.0.5.17 / 255.255.255.0

IP Address: 10.0.5.17

Subnet Mask: 255.255.255.0

CIDR Notation: /24

Network Address: 10.0.5.0

Broadcast Address: 10.0.5.255

Valid Host Range: 10.0.5.1 - 10.0.5.254

Calculation: 255.255.255.0 is /24. Applying the mask to 10.0.5.17 gives network 10.0.5.0. The last address in the /24 subnet is 10.0.5.255, so usable hosts are 10.0.5.1 through 10.0.5.254.

- Verification Using an Online Subnet Calculator

The values were verified using an online subnet calculator with IP address 10.0.5.17 and subnet mask 255.255.255.0 (/24).

Result: My manual answer matched the subnet calculator output: network address 10.0.5.0, broadcast address 10.0.5.255, and usable host range 10.0.5.1 - 10.0.5.254.