- IP Addressing and Subnetting Assignment

# 1. IP Address Classes

The following table shows the five traditional IPv4 address classes and a simple real-world example for each one. Classful addressing is an older method; modern networks normally use CIDR.

| Class | First Octet Range | Main Use | Simple Example |
| --- | --- | --- | --- |
| A | 0 - 127 | Very large networks | A large company or ISP-sized private network using addresses from 10.0.0.0/8. |
| B | 128 - 191 | Medium to large networks | A large college or university network using a range such as 172.16.0.0/16. |
| C | 192 - 223 | Small networks | A home WiFi router and devices using 192.168.1.0/24. |
| D | 224 - 239 | Multicast | IPTV, live video streaming, or routing protocols that send one packet to many receivers. |
| E | 240 - 255 | Experimental / reserved | Used for research or experimental purposes, not normal end-user device addressing. |

# 2. Network, Broadcast, and Valid Host Range

Given IP address: 192.168.10.45
Subnet mask: 255.255.255.0 (/24)

## Working Steps

Step 1: The subnet mask 255.255.255.0 means the first 24 bits are the network part and the last 8 bits are the host part.

Step 2: For the network address, make all host bits 0. Therefore, the network address is 192.168.10.0.

Step 3: For the broadcast address, make all host bits 1. Therefore, the broadcast address is 192.168.10.255.

Step 4: Valid host addresses are between the network and broadcast addresses, so the usable range is 192.168.10.1 to 192.168.10.254.

| Item | Answer |
| --- | --- |
| Network Address | 192.168.10.0 |
| Broadcast Address | 192.168.10.255 |
| First Valid Host | 192.168.10.1 |
| Last Valid Host | 192.168.10.254 |
| Total Usable Hosts | 254 |

# 3. Subnetting Plan for a Food Delivery Startup

Available network: 10.0.0.0/24
Required groups: 4
To make 4 equal subnets, we borrow 2 host bits because 2² = 4. The new prefix is /26, which gives subnet mask 255.255.255.192. Each /26 subnet has 64 total addresses and 62 usable host addresses.

| Group | Network Address | Subnet Mask | Usable Host Range | Broadcast |
| --- | --- | --- | --- | --- |
| Delivery Partners | 10.0.0.0/26 | 255.255.255.192 | 10.0.0.1 - 10.0.0.62 | 10.0.0.63 |
| Restaurants | 10.0.0.64/26 | 255.255.255.192 | 10.0.0.65 - 10.0.0.126 | 10.0.0.127 |
| Customers | 10.0.0.128/26 | 255.255.255.192 | 10.0.0.129 - 10.0.0.190 | 10.0.0.191 |
| Admins | 10.0.0.192/26 | 255.255.255.192 | 10.0.0.193 - 10.0.0.254 | 10.0.0.255 |

# 4. College WiFi Network - At Least 20 Subnets

Given network: 172.16.0.0/16
Required subnets: At least 20

## Calculation Steps

Step 1: Find the number of subnet bits needed. 2⁴ = 16, which is not enough. 2⁵ = 32, so we borrow 5 bits.

Step 2: Original prefix = /16. New prefix = 16 + 5 = /21.

Step 3: /21 in dotted-decimal form is 255.255.248.0.

Step 4: Host bits left = 32 - 21 = 11 bits.

Step 5: Total addresses per subnet = 2¹¹ = 2048. Usable hosts = 2048 - 2 = 2046 hosts per subnet.

| Item | Final Answer |
| --- | --- |
| Bits Borrowed | 5 bits |
| Number of Subnets Created | 32 subnets |
| New CIDR Prefix | /21 |
| New Subnet Mask | 255.255.248.0 |
| Usable Hosts per Subnet | 2046 |

# 5. Why 192.168.1.300 Is Invalid

The IP address 192.168.1.300 is invalid because every IPv4 octet must be between 0 and 255. The last octet is 300, which is greater than 255. Therefore, the device cannot use this address.

For a common 192.168.1.0/24 subnet, valid device addresses are normally from 192.168.1.1 to 192.168.1.254. For example, 192.168.1.100 would be a valid host address if it is not already being used by another device.