- IP Routing and Routing Protocols Assignment

1. Simple Network Diagram

Internet

|

WAN: 203.0.113.1

|

+----------------+

|     Router     |

| LAN:192.168.1.1|

+----------------+

/      |      \

/       |       \

/        |        \

Phone           Friend's Laptop       Smart TV

192.168.1.10         192.168.1.20      192.168.1.30

Gateway:             Gateway:          Gateway:

192.168.1.1          192.168.1.1       192.168.1.1

Device IP Addresses

| Device | IP Address | Default Gateway |
| --- | --- | --- |
| Router LAN Interface | 192.168.1.1 | - |
| Router WAN Interface | 203.0.113.1 | ISP |
| Phone | 192.168.1.10 | 192.168.1.1 |
| Friend's Laptop | 192.168.1.20 | 192.168.1.1 |
| Smart TV | 192.168.1.30 | 192.168.1.1 |

2. Manual IP Routing Simulation

Network Topology

Laptop ---- Router A ---- Router B ---- Friend's PC

192.168.1.10      10.0.0.1/30 10.0.0.2/30      192.168.2.20

IP Addresses

| Device | IP Address |
| --- | --- |
| Laptop | 192.168.1.10 |
| Router A LAN | 192.168.1.1 |
| Router A WAN | 10.0.0.1 |
| Router B WAN | 10.0.0.2 |
| Router B LAN | 192.168.2.1 |
| Friend's PC | 192.168.2.20 |

Routing Tables

Router A

| Destination Network | Subnet Mask | Next Hop |
| --- | --- | --- |
| 192.168.1.0 | 255.255.255.0 | Direct |
| 192.168.2.0 | 255.255.255.0 | 10.0.0.2 |

Router B

| Destination Network | Subnet Mask | Next Hop |
| --- | --- | --- |
| 192.168.2.0 | 255.255.255.0 | Direct |
| 192.168.1.0 | 255.255.255.0 | 10.0.0.1 |

Step-by-Step Packet Journey

Step 1

Laptop (192.168.1.10) wants to send data to Friend's PC (192.168.2.20).

Step 2

Laptop compares destination IP with its own subnet.

- Source Network = 192.168.1.0/24

- Destination Network = 192.168.2.0/24

Since they are different networks, the packet is sent to the default gateway (192.168.1.1).

Step 3

Router A receives the packet.

Router A checks its routing table:

| Destination | Action |
| --- | --- |
| 192.168.2.0/24 | Forward to 10.0.0.2 |

Router A forwards the packet to Router B.

Step 4

Router B receives the packet.

Router B checks its routing table:

| Destination | Action |
| --- | --- |
| 192.168.2.0/24 | Directly Connected |

Router B forwards the packet to Friend's PC.

Step 5

Friend's PC receives the packet and sends a reply back using the reverse path.

Final Path

Laptop

↓

Router A

↓

Router B

↓

Friend's PC

3. Routing Table for a Zomato-Style Delivery Network

Branch Locations

- Ahmedabad Branch

- Mumbai Branch

- Delhi Branch

Subnets

| Branch | Network |
| --- | --- |
| Ahmedabad | 192.168.10.0/24 |
| Mumbai | 192.168.20.0/24 |
| Delhi | 192.168.30.0/24 |

Routing Table

| Network Address | Subnet Mask | Next Hop | Interface |
| --- | --- | --- | --- |
| 192.168.10.0 | 255.255.255.0 | Direct | G0/0 |
| 192.168.20.0 | 255.255.255.0 | 10.1.1.2 | G0/1 |
| 192.168.30.0 | 255.255.255.0 | 10.1.1.6 | G0/2 |

Explanation

- Ahmedabad branch is directly connected.

- Traffic for Mumbai goes through Router M.

- Traffic for Delhi goes through Router D.

- Delivery orders and customer data can travel between branches using these routes.

4. Comparison of Static Routing and Dynamic Routing

| Static Routing | Dynamic Routing |
| --- | --- |
| Routes are configured manually. | Routes are learned automatically. |
| Suitable for small networks. | Suitable for large networks. |
| Less CPU and memory usage. | More CPU and memory usage. |

Static Routing

Advantages

- Easy to understand.

- Uses fewer router resources.

- More secure because routes do not change automatically.

Disadvantages

- Manual configuration required.

- Difficult to manage large networks.

- Network failure requires manual updates.

Example

A small office with only one internet connection can use static routing.

Dynamic Routing (RIP/OSPF)

Advantages

- Automatically discovers routes.

- Adapts to network changes.

- Easier to manage large networks.

Disadvantages

- Uses more CPU and RAM.

- More complex configuration.

- Generates routing update traffic.

Example

A food delivery company like Zomato with branches in many cities benefits from OSPF because routes automatically update if a link fails.

5. Sample OSPF Configuration

Router 1 Configuration

Router(config)# router ospf 1

Router(config-router)# network 192.168.1.0 0.0.0.255 area 0

Router(config-router)# network 10.0.0.0 0.0.0.3 area 0

Router 2 Configuration

Router(config)# router ospf 1

Router(config-router)# network 192.168.2.0 0.0.0.255 area 0

Router(config-router)# network 10.0.0.0 0.0.0.3 area 0

Explanation of Each Command

| Command | Description |
| --- | --- |
| router ospf 1 | Starts OSPF process ID 1. |
| network 192.168.1.0 0.0.0.255 area 0 | Advertises Router 1 LAN network into OSPF. |
| network 10.0.0.0 0.0.0.3 area 0 | Advertises router-to-router link. |
| network 192.168.2.0 0.0.0.255 area 0 | Advertises Router 2 LAN network. |
| area 0 | Places the network in OSPF backbone area. |

How OSPF Works

- Routers discover neighbors.

- They exchange routing information.

- Each router builds a routing table.

- Shortest paths are calculated automatically.

- If a link fails, OSPF finds a new route automatically.

Conclusion

IP routing allows data to move between different networks. Static routing is simple and suitable for small networks, while dynamic routing protocols such as OSPF are better for large networks because they automatically adapt to network changes.