- VLAN Configuration and Management Assignment

1. Network Diagram with VLANs

+------------------+

|      Switch      |

+------------------+

|       |       |

|       |       |

+------------+       |       +------------+

|                    |                    |

Laptop              Friend's Phone        Smart TV

192.168.20.10         192.168.20.20      192.168.10.10

VLAN 20               VLAN 20            VLAN 10

(Personal Device)    (Personal Device)   (Streaming Device)

VLAN Information

| Device | IP Address | VLAN | Purpose |
| --- | --- | --- | --- |
| Laptop | 192.168.20.10 | VLAN 20 | Personal Device |
| Friend's Phone | 192.168.20.20 | VLAN 20 | Personal Device |
| Smart TV | 192.168.10.10 | VLAN 10 | Streaming Device |

2. VLAN Configuration in Cisco Packet Tracer

Network Setup

VLAN 10

- PC1

- PC2

- PC3

VLAN 20

- PC4

- PC5

Port Assignment

| Device | Switch Port | VLAN |
| --- | --- | --- |
| PC1 | Fa0/1 | VLAN 10 |
| PC2 | Fa0/2 | VLAN 10 |
| PC3 | Fa0/3 | VLAN 10 |
| PC4 | Fa0/4 | VLAN 20 |
| PC5 | Fa0/5 | VLAN 20 |

Switch Configuration Commands

Create VLANs

Switch> enable

Switch# configure terminal

Switch(config)# vlan 10

Switch(config-vlan)# name STREAMING

Switch(config)# vlan 20

Switch(config-vlan)# name PERSONAL

Assign Ports to VLAN 10

Switch(config)# interface range fa0/1-3

Switch(config-if-range)# switchport mode access

Switch(config-if-range)# switchport access vlan 10

Assign Ports to VLAN 20

Switch(config)# interface range fa0/4-5

Switch(config-if-range)# switchport mode access

Switch(config-if-range)# switchport access vlan 20

Verify VLANs

Switch# show vlan brief

Ping Test

Successful Pings

PC1 → PC2  = Success

PC1 → PC3  = Success

PC4 → PC5  = Success

Failed Pings

PC1 → PC4  = Failed

PC2 → PC5  = Failed

Reason

Devices in different VLANs cannot communicate directly because each VLAN is a separate broadcast domain. Inter-VLAN routing is required for communication.

3. Moving a PC from VLAN 10 to VLAN 20

Assume PC3 is connected to Fa0/3.

Before Change

Switch# show vlan brief

VLAN 10 : Fa0/1 Fa0/2 Fa0/3

VLAN 20 : Fa0/4 Fa0/5

Connectivity Test

PC3 → PC1 = Success

PC3 → PC4 = Failed

Configuration Change

Switch(config)# interface fa0/3

Switch(config-if)# switchport access vlan 20

After Change

Switch# show vlan brief

VLAN 10 : Fa0/1 Fa0/2

VLAN 20 : Fa0/3 Fa0/4 Fa0/5

Connectivity Test After Change

PC3 → PC1 = Failed

PC3 → PC4 = Success

PC3 → PC5 = Success

Result

PC3 is now a member of VLAN 20 and can communicate with other devices in VLAN 20.

4. Automating Assignment of 10 Ports to VLAN 30

Create VLAN 30

Switch(config)# vlan 30

Switch(config-vlan)# name OFFICE

Assign Ports Fa0/1 to Fa0/10 Using Interface Range

Switch(config)# interface range fa0/1-10

Switch(config-if-range)# switchport mode access

Switch(config-if-range)# switchport access vlan 30

Verify Configuration

Switch# show vlan brief

Expected Output

VLAN 30 : Fa0/1 Fa0/2 Fa0/3 Fa0/4 Fa0/5

Fa0/6 Fa0/7 Fa0/8 Fa0/9 Fa0/10

Conclusion

VLANs are used to divide a switch into multiple logical networks. Devices within the same VLAN can communicate directly, while devices in different VLANs require a router or Layer 3 switch for communication. VLANs improve security, performance, and network management.