- NETWORK MONITORING AND TROUBLESHOOTING TOOLS ASSIGNMENT

Q1. Capture Network Traffic Using Wireshark

Objective: Capture about 2 minutes of your own network traffic while browsing Instagram or YouTube, save it as a .pcap file, and identify the top three protocols.

Install Wireshark from its official website. During Windows installation, allow Npcap to be installed because Wireshark uses it for packet capture.

Open Wireshark and select the active interface, usually Wi-Fi if the laptop is connected wirelessly.

Double-click the active interface to start capturing packets.

Open Instagram or YouTube in your browser and browse normally for about 2 minutes.

Return to Wireshark and click the red Stop button.

To see protocol statistics, open Statistics > Protocol Hierarchy and observe the protocols shown in the capture.

Write down the top three protocols you observe. The exact protocols depend on your traffic; do not copy example results without checking your capture.

Save the capture using File > Save As and choose the .pcap or .pcapng format requested by your trainer.

Q2. Test Spotify and Zomato Using ping

Open Command Prompt and run these commands one at a time:

ping www.spotify.com

ping www.zomato.com

Q3. Check Running Processes in Windows Task Manager

Press Ctrl + Shift + Esc to open Task Manager.

Click Processes. If Task Manager is in a compact view, choose More details if available.

Look for Chrome, WhatsApp, Microsoft Edge, or another browser-based application you use.

Note the process/application name and the Memory value shown beside it.

Q4. Scan Your Own Local Network Using an IP Scanner

Use this activity only on a network you own or have permission to scan, such as your own home Wi-Fi or an authorized college lab network.

Install Angry IP Scanner or Advanced IP Scanner from its official source.

Connect your laptop to your own Wi-Fi/network.

Open Command Prompt and run ipconfig. Note your IPv4 address and subnet mask so you know your local network range.

Open the scanner. Use only the local IP range belonging to your own network. Many scanners can detect the range automatically.

Start the scan and wait for it to finish.

From the results, identify at least two devices that you recognize or are authorized to document.

Record only the IP address and device/host name required by the assignment. Avoid scanning networks for which you do not have permission.

Q5. Three Hardware Tools Used in Network Troubleshooting.

| Hardware Tool | What It Does | When I Would Use It |
| --- | --- | --- |
| Cable Tester | Tests Ethernet cable continuity and pin wiring. | I would use it when a LAN cable is connected but the device has no network link or the cable may be faulty. |
| Crimping Tool | Attaches an RJ-45 connector to twisted-pair Ethernet cable. | I would use it when making or repairing an Ethernet patch cable. |
| Multimeter | Measures electrical values such as voltage, resistance and continuity. | I would use it to check power supplies, continuity or electrical problems affecting network equipment. |

Conclusion

Wireshark, ping, Task Manager and authorized IP-scanning tools help diagnose different parts of a network. Hardware tools such as cable testers, crimping tools and multimeters are useful when troubleshooting physical cabling and power-related problems.